Experiment No.: 3						Date:

Image Thresholding

Aim: Using OpenCV to try the various options of Image Thresholding

Theory:

Image thresholding is a technique in digital image processing that converts a grayscale image into a binary (black and white) image by assigning pixels a value of either black or white based on a predefined threshold value. It's a simple form of image segmentation used to separate objects from the background, making it a common first step for further analysis and interpretation of visual data.

There are many types of thresholding, like Simple thresholding, Adaptive thresholding, Otsu’s thresholding etc

Ins simple thresholding, if pixel value is greater than a threshold value, it is assigned one value (may be white), else it is assigned another value (may be black). The function used is cv2.threshold. First argument is the source image, which should be a grayscale image. Second argument is the threshold value which is used to classify the pixel values. Third argument is the maxVal which represents the value to be given if pixel value is more than (sometimes less than) the threshold value. OpenCV provides different styles of thresholding and it is decided by the fourth parameter of the function. Different types are:

• cv2.THRESH_BINARY

• cv2.THRESH_BINARY_INV

• cv2.THRESH_TRUNC

• cv2.THRESH_TOZERO

• cv2.THRESH_TOZERO_INV

In Adaptive Thresholding – In simple we used a global value as threshold value. But it may not be good in all the conditions where image has different lighting conditions in different areas. In that case, we go for adaptive thresholding. In this, the algorithm calculate the threshold for a small regions of the image. So we get different thresholds for different regions of the same image and it gives us better results for images with varying illumination

In the code below we use Matplotlib, which is a comprehensive library for creating static, animated, and interactive visualizations in Python. Matplotlib produces publication-quality figures in a variety of hardcopy formats and interactive environments across platforms. Matplotlib can be used in Python scripts, Python/IPython shells, web application servers, and various graphical user interface toolkits.

Procedure:

- Login to the Raspberry Pi

- Using the nano editor in the terminal write the following python code

sudo nano 3ThresholdImage.py

- To run the above code, use the command, and note down the observation

sudo python3 2ThresholdImage.py

Sample Code- 3ThresholdImage.py

import cv2

import numpy as np

from matplotlib import pyplot as plt

img = cv2.imread('Pictures/image_goa.jpg',0)

ret,thresh1 = cv2.threshold(img,127,255,cv2.THRESH_BINARY)

ret,thresh2 = cv2.threshold(img,127,255,cv2.THRESH_BINARY_INV)

ret,thresh3 = cv2.threshold(img,127,255,cv2.THRESH_TRUNC)

ret,thresh4 = cv2.threshold(img,127,255,cv2.THRESH_TOZERO)

ret,thresh5 = cv2.threshold(img,127,255,cv2.THRESH_TOZERO_INV)

titles = ['Original Image','BINARY','BINARY_INV','TRUNC','TOZERO','TOZERO_INV']

images = [img, thresh1, thresh2, thresh3, thresh4, thresh5]

for i in range(6):

plt.subplot(2,3,i+1),plt.imshow(images[i],'gray')

plt.title(titles[i])

plt.xticks([]),plt.yticks([])

plt.show()

Observation:

Conclusion: I was able to successfully try simple thresholding on the provided image.